Prilog za

IGP10-T22D2E-M3A2

IGP10		I/A Series, Electronic, Direct Connected Gauge Pressure Transmitter

T		Intelligent, Digital HART and 4 to 20 mA DC

22		Process Connection – 316L SS

Sensor – 316L SS

Sensor Fill Fluid – Silicone

Connection Type – ½ NPT External (male) Thread

D*		Span Limits (Range) – 0-21 bar

2**		PG 13.5 Conduit Connection, Aluminum Housing (With Electrical Safety Code E)

E		Electrical Safety – ATEX II 1 GD, EEX ia IIC, Zone 0

M3***		Painted Steel Bracket with Plated Steel Bolts, PG 13.5

A2**		Plastic PG 13.5 Cable Gland

*		Span limit must be at least 0-21 bar, can be higher but not above 0-40 bar

**		considering PG 13.5 thread is withdrawn from manufactoring in favor of M20x1.5 feel free to offer M20x1.5 conduit connection and cable gland

***	feel free to offer mounting bracket set „Painted Steel Bracket with Plated Steel Bolts“, M20x1.5 instead of PG 13.5, must include adapter fitting (M20x1.5 M to M8 F)