|  | ТECHNICAL ASSIGNMENT FOR SERVICE PROCUREMENT |
| --- | --- |

| Title of Technical Assignment for service procurement: | Calibration of laboratory instruments Mooney viscometers MV2000, MVB and Rheometer MDR 2000 manufactured by Alpha Technologies |
| --- | --- |
| Client (Block/Function): | Petrohemija Laboratory / FSK Laboratory, |

TABLE OF CONTENTS

1.	TECHNICAL ASSIGNMENT CLASSIFICATION  	2.

2.	SUBJECT OF TECHNICAL ASSIGNMENT AND LOCATION  	2

3.	DESCRIPTION OF AS-IS STATE 	2

4.	GENERAL PROVISIONS 	2

5.	SCOPE OF SERVICE AND TECHNICAL DESCRIPTION	3

6.	ТECHNICAL REQUIREMENTS 	3

7.	CONTRACTOR’S RIGHTS, OBLIGATIONS AND RESPONSIBILITIES..............……………………...3

8.	DEADLINE AND TIME SCHEDULE FOR SERVICE PROVISION  	3

9.	SERVICE QUALITY REQUIREMENTS AND METHOD OF QUALITY CONTROL 	3

10.	SERVICE ACCEPTANCE	3

11.	WARRANTY PERIOD 	3

12.	PRICE -  BILL OF QUANTITIES FOR MATERIALS AND LABOUR    	3

13.	HSE	4

14.	APPROVAL OF TECHNICAL ASSIGNMENT 	5

15.	APPROVERS - DiNIS	5

# TECHNICAL ASSIGNMENT CLASSIFICATION

| Теchnical Assignment stupulates procurement of service involving IT component | No |
| --- | --- |

| 1.5 Technical Assignment for other services realization |  |
| --- | --- |
| The Client is in possession of relevant technical documentation: | Yes (CE Declaration of Conformity for MV 2000, MDR and MDE 2000. ECLIPSE software with licenses incl. installation procedure floppy) |
| The relevant services defined in the Technical Assiggnment shall be carried out in accordance with the following principle: | - In line with precisely defined scope, unit prices per measurement units, and payment in accordance with the actually delivered quantities |

# SUBJECT OF TECHNICAL ASSIGNMENT AND LOCATION

Calibration and inspection of the laboratory instruments: Mooney viscometers type MV2000 and MVB, Rheometer type MDR 2000, with issuing a service report of Alpha technologies manufacturer’s service center. Calibration and inspection of the equipment shall be performed on the location of the Synthetic Rubber Plant (FSK), Elemir. The instruments are installed within the FSK Laboratory premises.

# DESCRIPTION OF AS-IS STATE

The laboratory instruments Mooney viscometers MV2000 and MVB, and Rheometer MDR 2000 were installed in 2005, 2016 and 2005 respectively, by a service provider authorized by the manufacturer Alpha Technologies.

| Prior to submitting a bid, the Bidder has to visit the location where the relevant service should be provided, and along with its bidding documets to submit a confirmation of site visit, duly signed by an authorized person of the Client: | No |
| --- | --- |

# GENERAL PROVISIONS

Calibration of the Mooney viscometers types MV2000 and MVB is performed in accordance with the recommendation of the ASTM D 1646-Standard Test Methods for Rubber-Viscosity, Stress Relaxation, and Pre-Vulcanization Characteristics (Mooney Viscometer). Calibration of the Mooney viscometer is done during its operation at test temperature normally used for analyses. The calibration is performed by using a weight providing a torsion moment equivalent to 100 Mooney units. The Industrial Reference Material IRM 241 is recommended to be used for checking up the Mooney viscometers after the calibration.

Calibration of the instrument Rheometer type MDR2000 is performed in accordance with the recommendation of the ASTM D 5289 standard test method for Rubber Property - Vulcanization Using Rotorless Cure Meters. The calibration is performed by reference torsion standard whose values are given in its calibration report. Checking up of the measuring temperature is confirmed by the comparison with a certified digital measuring system (digital reference thermometer).

# SCOPE OF SERVICES AND TECHNICAL DESCRIPTION

Regular annual calibration of laboratory instruments manufactured by Alpha Technologies: Mooney viscometers MV2000, MVB and Rheometer MDR2000. The service includes examination, calibration, and spare parts replacement as required. Calibration is to be performed once a year.

NOTE: Calibration standards and consumables required for performing the service are to be provided by the service provider.

# ТЕCHNICAL REQUIREMENTS

The service should include issuing of two Calibration Reports separately for each Mooney viscometer, MV2000, MVB and Rheometer MDR2000.

# CONTRACTOR’S RIGHTS, OBLIGATIONS AND RESPONSIBILITIES

The contractor shall perform the service of equipment in the client’s laboratories:

| Petrohemija Laboratory | Adres |
| --- | --- |
| FSK Laboratory | Industrijski put 13, Elemir |

# DEADLINE AND TIME SHEDULE FOR SERVICE PROVISION

The service must be completed by 31 December 2025.

# SERVICE QUALITY REQUIREMENTS AND METHOD OF QUALITY CONTROL

The quality control of the inspection and calibration of Mooney viscometers MV2000 and MVB, and Rheometer MDR2000 shall be in charge of an internal technical supervisor of the Instrument Maintenance Service.

# SERVICE ACCEPTANCE

The work performed will be confirmed by signing the Service completion record.

After completing the service in situ the contractor is obliged to prove the operational performances of the calibrated equipment and to deliver a service report electronically.

11.  WARRANTY PERIOD

The warranty period for the service should last at least 6 months.

12. TECHNICAL QUALIFICATION CRITERIA

| R.b. | Technical Qualification Criteria (TQC) | Supporting documentation (This is the same as the required TQC) |
| --- | --- | --- |
| 1. | The contractor must be an authorized service technician for the equipment in question. | Certificate of authorization / proof of manufacturer's authorization that the provider is an authorized service provider. |

# 12.  PRICE -  BILL OF QUANTITIES FOR MATERIALS AND LABOUR

| BILL OF QUANTITIES FOR MATERIALS AND LABOR / SERVICE SPECIFICATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item description | Units of Measurement | Quantity | Unit price | Total |
| 1. | Calibration of Mooney viscometer MV2000 | pcs | 1 |  |  |
| 2. | Calibration of Mooney viscometer MVB | pcs | 1 |  |  |
| 3. | Calibration of Rheometer MDR2000 | pcs | 1 |  |  |
| 4. | Travelling expenses | departure/arrival | 1 |  |  |
| Total: |  |  |  |  |  |

The offer should include price lists for: calibration of Mooney viscometers incl. temperature checks, calibration of Rheometer incl. temperature checks and torsion standard, travelling expenses.

# 13.  HSE

13.1 HSE risk level assessment

| No | Service | Taxonomy code | Taxonomy description | Risk assessment (L, M, H) |
| --- | --- | --- | --- | --- |
| 1. | Laboratory instrument calibration 600000130 | 581210 | Calibration/ standardization and inspection of laboratory equipment | 1(L)Low risk |

13.2 Appointment of persons responsible for the Contractor Management Procedure

The Sole Responsible Person  (SRP) shall be: Vanja Čorokalo, Head of the FSK Laboratory.

The person in charge for coordination and enforcement of the provisions of Agreement on Occupational Health and Safety, Environment Protection, and Fire Protection in the Company, TFU–328 contractual document (the НЅЕ person) shall be: Jaroslav Hrubik, OSH Risk Management Expert.

13.3 Requirements related to the number of HSE staff to be appointed by the Contractor

Table – Number of HSE staff per engagements

| Engagement description | Number of HSE staff |
| --- | --- |
| Contractor employes more than 30 workers (already covered by the HSE Agreement) | One HSE person per 30 workers |
| Contractor engages subcontractors | Each engaged company should have its own HSE person |
| Contractor and subcontractors | One HSE person per 20 workers |

If the Contractor independetly performs the contracted activities and engages over 30 employees (as defined in the mutually signed HSE Agreement), at least one HSE person must be appointed for every 30 employees (for example, if there are total of 134 employees engaged for the contract performance, there must be at least 5 HSE persons appoointed).

If the Contractor hires subcontractors for performance of the contracted ctivities, there are two possible cases:

- Each engaged subcontractor company must have a HSE person (it cannot be the same person for two or more companies) (for example, if the contractor engages five subcontractors, the total of six HSE persons would be required for the Contract performance, including the previously mentioned  restriction that the number of employees per HSE person may not exceed); or

- The contractor hires one HSE person for total of 20 subcontractor workers (in the case that on a small location, such as a fuel supply station, a collection-dispatch station, a well etc, a contractor hires five subcontractors for performing the contracted activities with a small number of workers, where the total number of the hired persons does not exceed 20) than the Contractor may engage one HSE person only to supervise the contracted activites regardless of the number of subcontractors.

The method of selecting the Contractor’s HSE persons shall be decided during the working meeting, аnd the result of the agreement shall be recorded in the minutes of kick-off meeting.

# APPROVAL OF TECHNICAL ASSIGNMENT

The Bidder is obliged to submit a Statement, signed and verified by an authorized person, confirming the acceptance of all the terms and requirements defined in the subject Technical Assignment.

# APPROVERS - DiNIS

| Authorized persons | Full name |
| --- | --- |
| Technical Assignment Author | Vanja Čorokalo |
| Project Manager | Branka Mijušković |
| Person responsible for HSE | Vladimir Stefanović |