|  | TECHNICAL ASSIGNMENT (SERVICES) |
| --- | --- |

| Title of the Technical Assignment for the procurement of services: | Chemical Washing and Mechanical Cleaning of Convection Sections |
| --- | --- |
| Client (Block/Function): | Ethylene Plant |

C O N T E N T S

1.	TECHNICAL ASSIGNMENT CLASSIFICATION	2

2.	SUBJECT AND LOCATION OF THE TECHNICAL ASSIGNMENT	2

3.	DESCRIPTION OF THE AS-IS STATE	2

4.	GENERAL PROVISIONS	2

5.	SCOPE OF THE SERVICE AND TECHNICAL DESCRIPTION	3

6.	TECHNICAL REQUIREMENTS	4

7.	RIGHTS, OBLIGATIONS, AND RESPONSIBILITIES OF THE CLIENT	4

8.	RIGHTS, OBLIGATIONS, AND RESPONSIBILITIES OF THE CONTRACTOR	4

9.	BASIS FOR THE SERVICE COMMENCEMENT	4

10.	RELEVANT SERVICE IMPLEMENTATION DEADLINE AND TIME SCHEDULE	5

11.	SERVICE QUALITY REQUIREMENTS AND METHOD OF QUALITY CONTROL	5

12.	REPORTING	5

13.	ACCEPTANCE OF THE PROVIDED SERVICE	5

14.	WARRANTY PERIOD	5

15.	TECHNICAL QUALIFICATION CRITERIA	6

16.	PRICE - BILL OF QUANTITIES FOR MATERIALS AND WORKS	7

17.	HSE	8

18.	APPROVAL OF THE TECHNICAL ASSIGNMENT	9

19.	ADDITIONAL NOTES	9

20.	APPENDICES	9

21.	APPROVERS – DiNIS (mandatory)	9

- TECHNICAL ASSIGNMENT CLASSIFICATION

| The Technical Assignment involves the procurement of a service with an IT component | No |
| --- | --- |

| 1.2.2 Technical Assignment for the service of execution of works with the service of obtaining technical conditions, approvals, and permits and/or preparation of technical documentation |  |
| --- | --- |
| The service of obtaining all necessary technical conditions, approvals, and permits is the obligation of: | the Client |
| The costs of all fees for obtaining technical conditions, approvals, and permits are borne by: | the Bidder |
| The service of preparation of technical documentation is an obligation of: | We have technical documentation |
| The service of technical control of the project for a construction permit is the obligation of: | the Client |
| The subject services defined in the Technical Assignment will be contracted according to the following principle | - According to precisely defined volumes, unit prices, units of measurement, and payment by actually completed quantities. |

1.5 Technical Assignment for the service of engaging expert supervision

- SUBJECT AND LOCATION OF THE TECHNICAL ASSIGNMENT

Chemical washing and mechanical cleaning of convection sections of superheaters and furnaces

The location of the works is HIP-Petrohemija Pančevo

- DESCRIPTION OF THE AS-IS STATE

| Prior to submitting the bid, the Bidder shall visit the location where the service is to be delivered and present, along with the tender documentation, a site visit confirmation, signed by the responsible person of the Client: | Yes |
| --- | --- |

- GENERAL PROVISIONS

The Contractor shall provide the specified services in compliance with the laws and by-laws, regulations, technical norms and standards of the Republic of Serbia, the standards of the profession, and the Client's requirements outlined in this Technical Assignment.

- SCOPE OF THE SERVICE AND TECHNICAL DESCRIPTION

In the upcoming downtime of the Ethylene Plant, it is necessary to do chemical washing and mechanical cleaning of convection sections of B-1002 A/B superheaters and F-1 A-H and F-2 A/B furnaces (removal of deposits and combustion products from the outside of the pipes)

Procedure of the execution of works:

- Chemical treatment

- Hydro mechanical cleaning

- Flushing and neutralization with high-pressure pumps

- Safe disposal of all waste and chemical materials

- Mechanical cleaning of pipes (outside) in the radiation section of B-1002 A/B superheater

Note:

During chemical washing, the Contractor must provide such a mode of operation, so as not to damage the refractory insulation in the floor and the ceiling of the superheater and the furnace. Wastewater from the washing process must be drained in such a way as not to damage the rest of the equipment.

So far, the washing has been done with oily or caustic soda (sodium hydroxide NaOH)

Convection section of F-1 A-H and F-2 A/B furnaces:

Pipe materials:

- SA-106 GR-B

- A-234 WPB

- A-213 T-11

- A-213 T-22

- A-213 TP 304H

- INCOLOY 800 GR II

Attached is the drawing 73F-1167-6 which defines the number of pipes, material and length by positions (rows), as well as which pipes are ribbed

Radiation section of B-1002 A/B superheaters:

- Pipe material SA-355 P11

Attached is the drawing 584-11 which defines the number of pipes, material and length by positions

Convection section of B-1002 A/B superheaters:

- Pipe material SA-106 GR-B

Attached is the drawing 584-01E which defines the number of pipes, material and length by positions (rows), as well as which pipes are ribbed

- TECHNICAL REQUIREMENTS

The Contractor is obliged to perform all works in a professional and quality manner, in accordance with the applicable technical regulations and standards for this kind of works.

- RIGHTS, OBLIGATIONS, AND RESPONSIBILITIES OF THE CLIENT

- to submit the available technical documentation;

- assembly/disassembly of the scaffolding

- disassembly/assembly of thermal insulation;

- person who will supervise the works

- Hiring a crane

- Hiring a forklift truck

- RIGHTS, OBLIGATIONS, AND RESPONSIBILITIES OF THE CONTRACTOR

- familiarization with the available technical documentation and on-site inspection, with the collection of all relevant data for the execution of works

- to appoint a person to manage all works on behalf of the Contractor, who will be in charge of contact and cooperation with the Client's representative

- to regularly keep the construction log

- To provide a qualified and trained workforce with experience in performing this type of works

- To provide the necessary tools for the work for the job in question (it is necessary to use the standard and non-sparking tools depending on the position and current conditions, which will be subsequently defined with the Client before the service commencement)

- Provide and use the necessary protective equipment and manage staff and work in accordance with the applicable PH procedures for engaging third parties

- Appointment of the Safety Coordinator, who would fully cooperate with the HSE Responsible Person on behalf of the Client

- The Contractor should undertake to complete the total work within the available overhaul period.

- When submitting the bid, it should supply the list of workers who will be engaged in these works.

- Define which chemical will be used for chemical washing

- Provide a statement/confirmation that the used chemical will not damage the pipes

- Submit a statement/confirmation that it will provide such a mode of operation, so as not to damage the refractory insulation in the floor and the ceiling of the superheater and the furnace.

- Provide correct and clean IBC containers in which liquid waste would be disposed of.

- BASIS FOR THE SERVICE COMMENCEMENT

The basis for the service commencement is a signed Contract/Purchase Order or the issued Letter of Intents.

- RELEVANT SERVICE IMPLEMENTATION DEADLINE AND TIME SCHEDULE

Chemical washing and mechanical cleaning is planned during the downtime of the Ethylene Plant in the period from 19 September 2026 to 31 October 2026.

All the listed services must be completed by 25 October 2026.

- SERVICE QUALITY REQUIREMENTS AND METHOD OF QUALITY CONTROL

Quality control of the performed works will be carried out by the expert supervision.

After washing, the pipes must be completely clean and without traces of deposits on them.

- REPORTING

The Contractor is obliged to regularly (daily) report to the Client on the progress of the performed works and to keep a construction log thereof, which will be verified by the expert supervision.

The Contractor shall also submit extraordinary reports in case of circumstances that stand in the way of the critical path in the project implementation or affect safety and security.

- ACCEPTANCE OF THE PROVIDED SERVICE

In order to sign the record, the Contractor must submit:

- Signed construction log for inspection

- WARRANTY PERIOD

The required warranty period for the completed works is 24 months.

The warranty period shall start from the date of handover of the performed service, that is, from the date of signing the Record on the Performed Services.

The Contractor is obliged to remove, at the Client's request and within the warranty period, all defects determined to have been caused by his fault (complaint).

The response time for complaints may not be longer than 48 hours from the time of its receipt.

- TECHNICAL QUALIFICATION CRITERIA

Table - Technical Qualification Criteria

| No. | Technical Qualification Criteria (TQC) | Supporting documentation (proving the compliance with the requested TQC) |
| --- | --- | --- |
| 1. | The company providing the service in question must be registered with the Business Register as the company for performing the activity that is the subject matter of the procurement | Submit the company registration excerpt (from the Business Registers Agency) |
| 2. | The Bidder/Contractor shall submit a Statement that it will perform the work that is the subject matter of procurement as follows: 1. independently; 2. by engaging subcontractors; 3. by entering into a consortium. Note: The Bidder shall indicate in which manner, among the above‑stated options, it participates in the tender for the procurement of the service in question. | 1. Statement of the independent performance of work, signed and stamped by an authorized person of the Bidder. 2. In case of submitting the bid with subcontractor(s), it is necessary to provide the name(s) of the subcontractor(s), the business and technical cooperation agreement for the work in question. 3. A consortium agreement and/or business and technical cooperation agreement obliging the participants to jointly perform the service in question and specifying the responsibility of each individual consortium member for the performance of contractual obligations. |
| 3. | Reference list in which the activities of chemical washing and mechanical cleaning in the petroleum/petrochemical industry are listed in the amount of at least RSD 2,000,000 | Certificates of the Client proving the allegations |
| … | … | … |

- In case that the stated certified staff is changed after submission of the documentation and delivery of the technical assessment, the Bidder is obliged to report the change in an urgent procedure. It is not allowed to make changes without making an agreement with the Client on the possible ways to overcome the occurred situation

- PRICE - BILL OF QUANTITIES FOR MATERIALS AND WORKS

Table - Display of data which a bill of quantities for materials and works should contain

| BILL OF QUANTITIES FOR MATERIALS AND WORKS / SERVICE SPECIFICATIONS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Item description | Unit of measurement | Quantity |  | Unit price | Total |  |
| 1 | B-1002 A | Set | 1 |  |  |  |  |
| 1.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials - Mechanical cleaning of pipes (outside) in the radiation section of B-1002 A/B superheater |  |  |  |  |  |  |
| 2 | B-1002 B | Set | 1 |  |  |  |  |
| 2.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials - Mechanical cleaning of pipes (outside) in the radiation section of B-1002 A/B superheater |  |  |  |  |  |  |
| 3 | F-1A | Set | 1 |  |  |  |  |
| 3.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 4 | F-1B | Set | 1 |  |  |  |  |
| 4.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 5 | F-1C | Set | 1 |  |  |  |  |
| 5.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 6 | F-1D | Set | 1 |  |  |  |  |
| 6.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 7 | F-1E | Set |  | 1 |  |  |  |
| 7.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 8 | F-1F | Set | 1 |  |  |  |  |
| 8.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 9 | F-1G | Set |  | 1 |  |  |  |
| 9.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 10 | F-1H | Set | 1 |  |  |  |  |
| 10.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 11 | F-2A | Set |  | 1 |  |  |  |
| 11.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| 12 | F-2B | Set |  | 1 |  |  |  |
| 12.1 | - Chemical treatment - Hydro mechanical cleaning - Flushing and neutralization with high-pressure pumps - Safe disposal of all waste and chemical materials |  |  |  |  |  |  |
| Total: |  |  |  |  |  |  |  |

Notes:

The Bidder cannot make any changes within the bill of quantities for materials and works.

The Bidder should not enter the unit price or the total price in the Technical Assignment.

The quantities specified in the Technical Assignment are maximum and non-binding

Invoicing and payment shall be based on the actual performed scope of work.

- HSE

The technical staff appointed by the Contractor for the works on the provision of this service is obliged to comply with all the requirements defined in the Appendix to this Technical Assignment – Contractor's HSE Obligations (Appendix 14 of the Instructions UP-52.00_09.01-001: Contractor's Management, version 1.1).

Work in confined space can be performed by employees of the Contractor who are trained for safe work in confined space. The Contractor's documentation and the fulfillment of this requirement (certificate of training) is checked at the kick-off meeting

17.1 Determination of the HSE risk level

Table - List of services

| No. | Services | Taxonomy code | Taxonomy description | Risk assessment (L, M, H) |
| --- | --- | --- | --- | --- |
| 1. | Chemical washing and mechanical cleaning of convection sections SAP: 600000114 | 521712 | Cleaning and degreasing of industrial floors | 12 (M) Moderate risk |

Final HSE risk level – refers to the subject of procurement as a whole:

      - Moderate risk (M).

17.2 Appointment of responsible persons in the process of "Contractor Management"

Directly responsible individual (DRI) shall be: Marko Bosanac, Maintenance Technician

The person responsible for coordinating and controlling the enforcement of the provisions of the Agreement on Occupational Safety and Health, Environmental Protection and Fire Protection in the Company, TFU – 328 to the contractual document (HSE Officer) shall be: Aleksandar Jovanov, OSH and IS Specialist.

- APPROVAL OF THE TECHNICAL ASSIGNMENT

The Bidder shall submit a Statement, signed and stamped by an authorized person, confirming the acceptance of all terms and requirements set forth in the relevant Technical Assignment.

The bids that lack evidence of fulfilment of requirements outlined in the general and technical parts of the bid shall not be considered.

# ADDITIONAL NOTES

If the Bidder determines that there are deficiencies within the relevant Technical Assignment or other tender documentation, it must notify the contact persons specified in the Invitation to Bids prior to the expiry of the bid submission deadline.

- APPENDICES

- Drawings of convection and radiation sections

73F-1167-6

584-01 E

- APPROVERS – DiNIS (mandatory)

Table – Approvers of the Technical Assignment

| Responsible persons | Name and surname |
| --- | --- |
| Author of the Technical Assignment: | Đurica Kolarov |
| Project Manager: | Marko Janković |
| HSE responsible person | Vladimir Stefanović |